BCT 2408 Lab1
Enoch Teddy
SCT212-0078/2021
E1: H&P
Given:

- The old system has a 5% faster clock →
ClockTime_old = 0.95 × ClockTime_new

- 30% of the instructions in the old system are loads/stores
LoadStore_old = 0.30 × InstrCount_old

- Optimized version reduces load/store by 1/3 →
LoadStore_new = (2/3) × LoadStore_old

- Non-load/store instruction count remains the same

- All instructions take 1 clock cycle → CPI = 1

CPU Time = Instruction Count × CPI × Clock Time
CPUTime_old = InstrCount_old × ClockTime_old = InstrCount_old × 0.95 × ClockTime_new
InstrCount_new = (0.70 × InstrCount_old) + ((2/3) × 0.30 × InstrCount_old)

= 0.70 × InstrCount_old + 0.20 × InstrCount_old

= 0.90 × InstrCount_old
CPUTime_new = InstrCount_new × ClockTime_new = 0.90 × InstrCount_old × ClockTime_new

Speedup = CPUTime_old / CPUTime_new

= (0.95 × InstrCount_old × ClockTime_new) / (0.90 × InstrCount_old × ClockTime_new)

= 0.95 / 0.90

= 1.06
Conclusion: The optimized version is faster, with a 6% speed improvement.

E2: H&P
Part 1: Percentage of Loads to Eliminate
Given:

- New instruction increases clock period by 5%
CycleTime_new = 1.05 × CycleTime_old

- CPI remains the same
CPI_new = CPI_old

ExecutionTime = Instruction Count × CPI × Cycle Time

InstrCount_new × 1.05 × CycleTime_old = InstrCount_old × CycleTime_old

InstrCount_new × 1.05 = InstrCount_old
⇒ InstrCount_new = 0.95 × InstrCount_old

R = number of instructions removed
InstrCount_new = InstrCount_old – R
⇒ InstrCount_old - R = 0.95 × InstrCount_old
⇒ R = 0.05 × InstrCount_old
LoadInstr_old = 0.228 × InstrCount_old
R = 0.05 × InstrCount_old = (0.05 / 0.228) × LoadInstr_old ≈ 0.22 × LoadInstr_old

You need to eliminate at least 22% of the load instructions to break even in performance.

Part 2: Non-Replaceable Example

Consider the following instruction pair:
LOAD R2, 0(R6)
ADD R2, R2, R2

Suppose:

- Register R6 points to address 1000

- Memory[1000] = 4

- Initial value of R2 is 47

Actual result:

- LOAD puts 4 in R2

- ADD computes 4 + 4 = 8

If replaced by:
ADD R2, 0(R6)
Then the addition becomes 47 + 4 = 51, using the old R2 value.

This is incorrect.
The transformation is not valid when the same register is used as both source and destination, because it changes the meaning of the program.