BCT 2408 Lab 4
Enoch Teddy
SCT212-0078/2021
E1:

a)What is the tag size?

- Block offset bits = log2(64) = 6 bits

- Cache has 256 KB / (64 × 4) = 1024 sets → log2(1024) = 10 bits for index

- Tag bits = 32 - (10 + 6) = 16 bits

### b) How much faster is the ideal machine (with no cache misses)?

- Effective memory references per instruction:

  - 1 instruction fetch + 0.5 data accesses = 1.5 memory accesses

- Miss penalty = 25 cycles

- Miss rate = 2% = 0.02
⇒ Average stall cycles per instruction =
1.5 × 0.02 × 25 = 0.75

- Actual CPI = 1 + 0.75 = 1.75

- Speedup = 1.75 / 1.0 = 1.75× faster

E2:

### a) Write-through cache (with write allocate)

- Read hit: 0 words to memory

- Read miss: 2 words fetched

- Write hit: 1 word written

- Write miss: 2-word fetch + 1-word write = 3 words

Total average words transferred per operation:

= 0.7125×0 + 0.0375×2 + 0.2375×1 + 0.0125×3

= 0 + 0.075 + 0.2375 + 0.0375

= 0.35 words per operation

Memory bandwidth usage = 0.35 × 10⁹ words/sec
Fraction of bandwidth used = 0.35

### b) Write-back cache (with write allocate)

- Read hit and write hit: 0 memory traffic

- On a read/write miss:

  - If evicted block is dirty (30%), write 2 words back + fetch 2 words

  - If clean (70%), just fetch 2 words

→ Expected words per miss = 0.7×2 + 0.3×4 = 2.6 words

Average words transferred:

= 0.0375 × 2.6 + 0.0125 × 2.6

= 0.0975 + 0.0325

= 0.13 words per operation

Memory bandwidth usage = 0.13 × 10⁹ words/sec
Fraction of bandwidth used = 0.13

Write-back caching reduces memory bandwidth usage by more than half compared to write-through under these conditions.

E3:

Solution

General Performance Equation:

CPU_Time = Instruction_Count × CPI × Clock_Cycle_Time

CPI = CPI_base + Average_Stall_Cycles_per_Instruction

Step 1: Base CPI (no misses)

- Loads (read hit) take 1 cycle

- Stores (write hit) take 2 cycles

- All other instructions take 1 cycle

So:

CPI_base = 0.26 × 1 + 0.09 × 2 + 0.65 × 1 = 1.09

Step 2: Cache Miss Penalty – Stall Cycles

a. Write-Through Cache

- Instruction cache:

Miss rate = 0.005, penalty = 50

→ Instruction stall = 0.005 × 50 = 0.25

- Data cache (read + write):

Miss rate = 0.01

Total data access fraction = 0.26 (loads) + 0.09 (stores) = 0.35

→ Data stall = 0.01 × 0.35 × 50 = 0.175

Stall_Cycles_Write_Through = 0.25 + 0.175 = 0.425

CPI_Write_Through = 1.09 + 0.425 = 1.515

b. Write-Back Cache

Here, on a data miss:

- 50% chance the evicted block is dirty → write it back (50 more cycles)

- Effective penalty = average of 50 (clean) and 100 (dirty) = 75 cycles

So for loads and stores:

Average data stall = 0.01 × [0.26 × 75 + 0.09 × 75] = 0.01 × 0.35 × 75 = 0.2625

Instruction miss stall = 0.005 × 50 = 0.25

Stall_Cycles_Write_Back = 0.25 + 0.2625 = 0.5125

CPI_Write_Back = 1.09 + 0.5125 = 1.6025

Conclusion: The write-back system is approximately

((1.6025 - 1.515) / 1.515) ≈ 5.77% slower — roughly 6% performance drop due to increased stall time from dirty block handling.