BCT 2408 Lab 5
Enoch Teddy
SCT212-0078/2021

Solution
E1:

### a) Naive Access – Cache Misses

#### Cold Misses:

- On every 16th iteration, new 64B cache line (16 floats) is loaded:

  - X[i] and Y[i] → 4096/16 = 256 accesses each

  - Total cold misses: 2 × 256 = 512

#### Conflict Misses:

- Direct-mapped: X and Y share overlapping cache lines

- After each load Y[i], the corresponding line of X is evicted

- Thus:

  - Each store X[i] → misses (evicted by Y[i])

  - The 15 elements of Y that were loaded after cold misses → evicted

- Total conflict misses: 4096 (stores) + 15/16 × 4096 (Y loads) = 7936

#### Total:

- Total misses = 512 (cold) + 7936 (conflict) = 8448

- Miss rate = 8448 / 12288 = 0.6875 or 68.75%

### b) Software Optimization

#### Option 1: Interleave X and Y

- Store X[0], Y[0], X[1], Y[1], ... in memory alternately

- Each 64B line can now hold 8 Xs and 8 Ys

- Cold misses: 4096 / 8 = 512

- Conflict misses: 0 (no X/Y interference)

- Miss rate = 512 / 12288 = 0.0417 (4.17%)

#### Option 2: Separate X and Y in memory by >16KB

- This prevents cache line conflicts

- Cold misses: 256 each → 512

- Conflict misses: 0

- Miss rate = 4.17%

### c) Hardware Optimization

#### Option 1: Double Cache Size to 32KB

- Cache now fits both X and Y simultaneously

- Cold misses: 512

- Conflict misses: 0

- Miss rate = 4.17%

#### Option 2: Make Cache 2-Way Set Associative

- Allows X and Y to map to same set without evicting each other

- Cold misses: 512

- Conflict misses: 0

- Miss rate = 4.17%

#### Option 3: Increase Block Size

- Say block size = 128B → 32 floats

- Cold misses: 4096 / (16×z) → fewer

- Conflict misses: worsen dramatically (now evictions occur more frequently)

- Total: up to 8192 misses

- Miss rate ≈ 66.67%

#### Option 4: Add Next-Line Prefetcher

- Prefetch Y[i+1] after Y[i] is loaded

- Cold misses reduced: 256

- Conflict misses: 8064

- Miss rate = (256+8064)/12288 ≈ 66.7%

#### Option 5: Add Victim Cache (small fully associative buffer)

- Cold misses: 256

- Conflict misses: 4096 (Y evicts X, but store hits in victim cache)

- Total: 4352

- Miss rate = 4352 / 12288 = 35.42%